# BeamNetUS FY 26 Proposal

Optimized Irradiation of Target Material for Dynamic Nuclear Polarization Experiments at Jefferson Lab

User Principal Investigator (PI):

Elena Long

Department of Physics and Astronomy

University of New Hampshire

Associate Professor

DeMeritt Hall 237A
9 Library Way

Durham, NH 03824

E: elena.long@unh.edu

T: +1 (603) 862-5312

User Co-Principal Investigator (Co-PI):

Karl Slifer

Department of Physics and Astronomy

University of New Hampshire

Professor

DeMeritt Hall 237A
9 Library Way

Durham, NH 03824

E: karl.slifer@unh.edu

T: +1 (603) 722-0695

User Co-Principal Investigator (Co-PI):

S. Nathaly Santiesteban

Department of Physics and Astronomy

University of New Hampshire

Assistant Professor

DeMeritt Hall 237A
9 Library Way

Durham, NH 03824

E: Nathaly.Santiesteban@unh.edu

T: +1 (603) 781-0171

## Abstract

Dynamic Nuclear Polarization (DNP) has been used as a technique for creating spin-polarized targets for studying the spin structure of nucleons and nuclei for decades. Several upcoming experiments at Jefferson Lab use such targets, including the E12-13-011 and E12-15-005 experiments to measure the tensor spin observables b1, Azz, and T20 that will use DNP on deuterated ammonia (ND3) to achieve a high tensor polarization of 30%. This new tensor target requires optimization of the material’s preparation. Historically such targets have been prepared by performing a ‘warm irradiation’ in liquid argon, though ND3 targets benefit significantly from an additional ‘cold irradiation’ in liquid helium. Cold irradiation is typically achieved parasitically during experimental data taking, which has not allowed for an in-depth study of the mechanisms by which it improves polarization. We intend to study the effects of cold irradiation by varying the temperature and beam dose on target materials to find an optimal configuration that maximizes both vector and tensor polarization of the material. The Jefferson Lab Upgraded Injector Test Facility (UITF) is an unique facility for this effort as JLab also runs a dedicated Target Test Lab with an on-site DNP target within walking distance of the UITF, which allows the materials to be immediately tested following irradiation to determine the maximum polarization from each combination of temperature and dose, which can then be used to further improve the following irradiation run. Once optimal conditions are found, they can be utilized to improve the maximum polarization achieved for all future DNP experiments at Jefferson Lab and elsewhere. We request 4 weeks of 8-10 MeV, 10 µA rastered electron beam at the UTIF to study and optimize the radiation dose and temperature on 250 mg of material used for Dynamic Nuclear Polarization, and to perform the optimized dose on an additional 250 mg of material for upcoming experiments at Jefferson Lab. The materials will be solid high purity ammonia (both NH3 and ND3) and butanol held at cryogenic temperatures in a beamline-ready variable temperature cryostat capable of cooling down to 2 K that is currently under construction at Jefferson Lab.

## 1. Introduction

Solid dynamic nuclear polarization (DNP) targets are often used in medium-energy experimental nuclear physics to study the spin-dependent structure of nuclei and nucleons. These targets typically operate within a magnetic field of ~5 T and at a temperature of ~1 K. At thermal equilibrium in these conditions, typical polarizations are ~0.5% for protons and ~0.1% for deuterons. However, unpaired electrons have a high thermal equilibrium polarization of >99%. DNP is a mechanism by which the large polarization of unpaired electrons is transferred to nuclei, with DNP proton polarization typically reaching >85% and DNP deuteron polarization reaching >10%.

Polarization experiments using solid DNP targets often use ammonia as the target material of choice for polarized protons (NH3) or polarized deuterons (ND3), as it has been found to both highly polarize and is among the most radiation resistant of DNP target materials [1,2]. To introduce unpaired electrons (also referred to as free radicals or paramagnetic centers) into the target material, they are pre-irradiated with an 8-20 MeV particle beam of up to 10 µA. During this pre-irradiation, the material is typically submersed in liquid argon (87 K). However, ND3 is unique in that large (>40%) polarizations can only be achieved if an additional ‘cold irradiation’ occurs while the material is below liquid helium temperatures (<4 K).

While there have been studies done to determine the radicals formed within such material [3,4], fundamental questions remain for finding the optimal irradiation conditions for maximizing ND3 polarization: What causes the higher ND3 polarization to be achieved after the additional ‘cold irradiation’, and why does it differ from NH3? What maximum temperature is required to create the cold irradiation? Is there an optimal temperature to create the polarizing radicals that avoids creating extraneous radicals that leads to depolarization? How long does the high polarization achieved from a cold irradiation last when the irradiated material is stored under liquid nitrogen (77 K)?

The proposed work aims to answer these questions by utilizing the Jefferson Lab Upgraded Injector Test Facility (UITF). The UITF can provide an electron beam of up to 8 MeV at up to 10 µA that is ideal for such a study. In addition, Jefferson Lab hosts a Target Test Lab within walking distance of the UITF where irradiated material can be tested immediately after irradiations are completed to determine the maximum polarization achieved by various temperature and dose conditions. The results of this study will provide the optimal material preparation technique for ammonia, particularly ND3, that will allow upcoming experiments at Jefferson Lab using such targets to have maximum polarizations earlier in experimental running, which will increase the statistics taken during the experiments, leading to higher quality results. This includes the upcoming E12-13-011 and E12-15-005 experiments to measure the tensor spin observables b1, Azz, and T20, which require a large initial vector polarization to achieve the large tensor polarization of ~30% required for these experiments to run. Additionally, once optimal irradiation conditions are found, we will use those conditions to prepare the material required for running the upcoming E12-13-011 and E12-15-005 experiments.

## 2. Scientific or Technical Objectives

The proposed work has two primary objectives: 1) To determine the optimal temperature and dose required for achieving maximum polarization in materials used for DNP targets, and 2) to use these results to prepare target material for upcoming DNP experiments at Jefferson Lab.

The DNP process requires polarizable material to contain free radicals, which are induced in NH3 and ND3 by bombarding the nuclei with an 8-20 MeV electron beam. Previous electron spin resonance (ESR) studies indicate that the primary radicals created during this process are ṄH2 and ṄD2, though atomic nitrogen and an additional Hydrazine radical have been found in irradiated ND3 [4,5].

In typical DNP polarized target experiments with a magnetic field of 5 T and a temperature of 1 K, NH3 has been shown to polarize well (>90%) after an irradiation of approximately 1017 e-/cm2 in liquid argon at 87 K, but ND3 requires an additional ‘cold irradiation’ of at least 1015 e-/cm2 at 1 K along with annealing to polarize well (>20% vector) [6], though the optimal amount of cold irradiation has yet to be precisely determined. Each of the studies cited so far mention that further work is needed to fully understand the mechanism by which the additional cold irradiation is required to achieve high polarizations in ND3 but not in NH3.

It is also currently unknown if the cold dose can be achieved at normal liquid helium temperature of 4.2 K or higher, or whether further reduction in temperature down to 2 K or 1 K is required. The first two weeks of the proposed work will undertake such a study by systematically varying the temperature from 87 K to 2 K, varying the irradiation dose from 1015 to 1018 e-/cm2 to determine optimal polarizing conditions.

The PIs will provide 250mg of material to be irradiated for the optimization tests during the first two weeks of beam time, as well as an additional 250mg of material to be irradiated under the optimal conditions during the second two weeks of beam time that will be used for the upcoming E12-13-011 and E12-15-005 experiments to measure tensor spin observables. The PIs have previous expertise in creating NH3, ND3, and deuterated butanol target material, and have done previous irradiations under liquid argon at the Medical-Industrial Radiation Facility (MIRF) accelerator at NIST, which has a 12 MeV, 10 µA electron beam.

The temperature of the material being irradiated will be controlled using a variable temperature cryostat that is currently being constructed by Jefferson Lab’s Target Group. It is capable of achieving arbitrary temperatures from 300 K down to 2 K, and is being built specifically for use at the UITF. The Jefferson Lab Target Group is led by Chris Keith, who is in active collaboration with the PIs for this project.

The primary experimental observables are the temperature and dose performed on each set of material, the maximum positive and negative polarizations that can be achieved on each set of material following irradiation, and the width of microwave frequencies used to enhance polarization. These measurements will be done on-site at the Jefferson Lab Target Test Lab, which has a 5T/1K DNP target within walking distance of the UITF. The PIs and our workforce from the University of New Hampshire operate a similar 5T/1K DNP target, and our effort will be split on-site at Jefferson Lab between doing the irradiations at the UITF and then immediately measuring the results from each irradiation at the Target Test Lab.

## 3. Experimental Details

The proposed work is best suited for the Jefferson Lab Upgraded Injector Test Facility (UITF). The 8 MeV, 10 µA electron beam matches both the energy and current required for this project. Additionally, the variable temperature cryostat currently under construction is being built specifically for use at the UITF, and the additional facilities of the Jefferson Lab Target Test Lab are required for rapid testing and measurement of results from the irradiation process. This includes a DNP target test stand that operates a superconducting magnetic with a 5 T field, and an evaporative helium refrigerator that reaches temperatures of 1 K, a 20 W microwave source use to drive the DNP process, and a nuclear magnetic resonance (NMR) system that measures the amount of polarization achieved within the irradiated material, along with a ‘target stick’ that holds the irradiated material samples, waveguides for microwave transmission, and NMR coils for measurement. We expect to be ready for beam operations by June 2026, which includes the completion and commissioning of the variable temperature cryostat and the DNP target test stand.

The variable temperature cryostat can hold a sample size of up to 40 cm3, which can be rotated to ensure uniform irradiation of the material. The PIs have previously used a similar volume for warm 87 K irradiations at the MIRF accelerator, where the sample holder was a cylinder with a length of 6.5 cm and radius of 1.42 cm, corresponding to a volume of 41.7 cm3 and a mass of approximately 20mg. The beam interaction area was 6.42 cm2, and is expected to be similar at the UITF. The energy deposited in the material drops off rapidly, so each irradiation must be done twice (once on the front side, and once on the back side) to uniformly irradiate the material.

The warm irradiation cycles will produce a dose on the order of 1017 e-/cm2 at 10 µA, which requires that each side of the material be irradiated for 2.85 hours for a total of 5.70 hours for each batch of material.

10 μA ×2.85 hr / 6.42 cm2=6.42×1017 e-6.42 cm2=1×1017 e-cm2

The cold irradiation cycles will produce a dose on the order of 5×1015 e-/cm2 at 1 µA. The lower current for the cold irradiation cycles is to reduce the heat load on the target material so that temperatures as low as 2 K can be maintained during beam irradiation. This requires that each side of the material be irradiated for 1.42 hours for a total of 2.84 hours for each batch of material.

1 μA ×1.42 hr / 6.42 cm2=3.21×1016 e-6.42 cm2=5×1015 e-cm2

The irradiation of each batch of material is expected to take, on average, 8.6 hours. This will be taken during the daily operation of the UITF.

Once irradiation of a batch is completed, the material will be taken to the Target Test Lab to measure the effectiveness of the irradiation. This will consist of a slow microwave frequency sweep of ±0.5 GHz around the central electron Larmor frequency, which is approximately 140 GHz at 5T. The sweep will step in intervals 0.01 GHz for 1 minute per step, leading to a total sweep time of 1.7 hours. During the sweep, the polarization will be measured multiple times at each step, allowing for a plot of the change in polarization vs. frequency to be created. From this plot, the maximum polarizing frequencies can be obtained for each batch of material by finding the frequencies where maximum and minimum ∆Pt occur. The microwave frequency will then be set to the value corresponding to the maximum positive polarization frequency found from the frequency sweep, and a 10 hour long spin-up will be taken to determine the maximum polarization of the material batch. The maximum polarization value will then be used to determine whether the parameter being adjusted in the irradiations should increase or decrease, and the process repeated. While irradiations are being performed, the material batch already in the DNP target will be allowed to relax for approximately 8 hours by turning off the microwaves in order to measure the nuclear T1 relaxation time of the batch.

The first week of beam running will be dedicated to finding the optimal temperature for cold irradiations. This will be done similar to a binary search to efficiently find the temperatures above and below which the cold irradiations are effective. Starting with the first batch at 2 K, we will double the temperature in each subsequent batch until the effects of cold irradiation are no longer seen in the material. Once we have gone above the temperature required for cold irradiation, we will go back towards the previous temperature by a factor of ½ of the previous interval. For example, if the cold irradiation radicals required for large maximum polarization start forming at 6.5 K, then the cycle of temperatures will be 2 K (large P), 4 K (large P), 8 K (small P), 6 K (large P), 7 K (small P), 6.5 K (large P), 6.75 K (small P).

Once the first week of irradiations is completed, the cold irradiation temperature that gave the largest maximum polarization will be used to study the effect of changing the amount of cold irradiation dose on the material during the second week of irradiations. This will again be achieved by a binary search method, doubling the dose for each batch of material. When increasing dose, the majority of the same material that has already been irradiated (minus a fraction of material being used to measure the T1 relaxation rate) will be cold irradiated for a second time in order to reduce the beam time required for this test. Following each batch’s irradiation, it will then be tested for optimal polarization frequencies, maximum polarization value, and T1 relaxation rate as described earlier.

The conditions that led to the largest maximum polarizations will then be used to irradiate ND3 material during the final two weeks that will be used in the upcoming E12-13-011 and E12-15-005 experiments at Jefferson Lab, as well as a small amount of NH3 and d-butanol that can be used for future experiments. Following the beam time proposed here, samples from each test batch of material will have their electron spin resonance (ESR) measured to further confirm the radicals present in each batch of material.

## 4. Beam and Facility Requirements

The proposed work will utilize the UITF at an energy of 8 MeV with currents from 1-10 µA. The beam spot size is expected to have a radius on the order of 14 mm, which can be achieved by a combination of rastering and adding a thin ‘beam spreading’ window of aluminum where the beam enters the cryostat. The placement of the Jefferson Lab Target Group’s irradiation cyrostat on the UITF beamline, both externally and internally, is shown in Fig. 1.

Fig. 1. Schematic of the variable temperature cryostat placed at the end of the UITF beamline (left), and a cross-section of the interior of the cryostat highlighting the rotatable hexagonal cage that will hold the material to be irradiated (right). The white and red arrow indicates the beam direction. Figures courtesy of Casey Flanagan, Daniel Akers, and Chris Keith.

The target materials will be ~1 mm radius solidified spheres of ND3, NH3, and deuterated butanol. The material will be created at the University of New Hampshire (UNH) using our existing material solidification cryostat that has already generated multiple >2 kg of similar target materials, and will be transported to Jefferson Lab for the irradiation tests. The material will be held under liquid nitrogen in dewars provided by both UNH and the Jefferson Lab Target Group.

No changes in personnel are expected between proposal submission and project completion. The workforce for this project will operate in three shifts. The first shift will be available for helping to run the beam operations and variable temperature cryostat. The second shift will transfer the freshly irradiated material to the DNP target test stand and run the microwave frequency sweep test and begin the maximum polarization test. The third shift will complete the maximum polarization test, analyze the preliminary results, and communicate them to the following beam operations shift.

The primary potential risk to project completion would be if the irradiation cryostat or the DNP target test lab at Jefferson Lab were not fully commissioned by the start of beam running. However, the risk from the irradiation cryostat is low as it is currently already approximately 50% completed. UNH students have already spent time at Jefferson Lab to aid in its construction, and if it falls behind schedule we can again send workforce to Jefferson Lab to ensure that it is ready in time. Similarly, UNH students, post docs, and faculty can visit Jefferson Lab before beam operations to aid the Jefferson Lab Target Group in commissioning the DNP target test stand if that falls behind schedule. Both the UNH group and the Jefferson Lab Target Group have successfully commissioned such DNP targets in the past, and all related personnel have expertise in such targets.

| Day -5 - Day 0 (Setup) | Installation of the irradiation cryostat in the UITF beamline. This work can be completed before the start of the proposed project. |
| --- | --- |
| Day 1 - Day 7 | Temperature testing of cold irradiation on ND3. Each day will have 5.7 hours of warm irradiation at 87 K with 8 MeV, 10 µA beam followed by 2.9 hours of cold irradiation at various temperatures down to 2 K with 8 MeV, 1 µA beam. Once irradiated, material will be transferred to the DNP target test stand to perform a microwave frequency sweep test and maximum polarization test. During the following day’s beam operations, the material in the DNP test stand will have its T1 relaxation time measured. |
| Day 8 - Day 14 | Dose testing of cold irradiation on ND3. Each material batch will start with 5.7 hours of warm irradiation at 87 K with 8 MeV, 10 µA beam followed by increasing time of cold irradiation with a 1-10 µA beam, optimized to reduce beam time while maintaining the required temperature found during the previous week. Once irradiated, material will be transferred to the DNP target test stand to perform a microwave frequency sweep test and maximum polarization test. During the following day’s beam operations, the material in the DNP test stand will have its T1 relaxation time measured. |
| Day 15 - Day 27 | Irradiation of ND3 for upcoming Jefferson Lab experiments. 250mg of material will be irradiated following the optimal dose and temperature found during the previous two weeks. Each material batch will start with 5.7 hours of warm irradiation at 87 K with 8 MeV, 10 µA beam followed by the optimized time of cold irradiation with a 1-10 µA beam determined in the previous week. |
| Day 28 | Irradiation of NH3 and deuterated butanol. The batch of NH3 will be irradiated for 5.7 hours at 87 K with 8 MeV, 10 µA beam (it does not require cold irradiation). The batch of deuterated butanol will be irradiated for 4 minutes at 87 K with 8 MeV, 10 µA beam. |

Table 1: Proposed experimental schedule

## 5. Broader Impact

The proposed work will provide training for the next generation of nuclear physicists in United States. Multiple graduate students will gain experience on running the irradiations and DNP target for the proposed tests, and the material generated during the tests will be used for the upcoming Jefferson Lab experiments E12-13-011 and E12-15-005 that are the primary topics for their Ph.D. theses. The work done in this proposal will also be included as a chapter of the students’ theses. The results of the work will be written up and published, with a target journal of Nuclear Instrumentation and Methods in Physics Research A. It will also be presented at national and international conferences, with targeted conferences of the Workshop on Polarized Sources, Targets, and Polarimetry (PSTP) and the International Spin Physics Symposium (SPIN). Results from this work will provide crucial new understanding on the cold irradiation process, as well as a recipe for optimal irradiation procedures that can be utilized by every following DNP target experiment that uses ND3 as a target material. This project directly supports both the upcoming E12-13-011 and E12-15-005 experiments at Jefferson Lab to measure the tensor structure of nuclei and nucleons.

## Appendix I: References

[1]	D. G. Crabb and W. Meyer, SOLID POLARIZED TARGETS FOR NUCLEAR AND PARTICLE PHYSICS EXPERIMENTS, Annu. Rev. Nucl. Part. Sci. 47, 67 (1997).

[2]	St. Goertz, W. Meyer, and G. Reicherz, Polarized H, D and 3He targets for particle physics experiments, Prog. Part. Nucl. Phys. 49, 403 (2002).

[3]	W. Meyer, K. H. Althoff, W. Havenith, O. Kaul, H. Riechert, E. Schilling, G. Sternal, and W. Thiel, Dynamic deuteron polarization in irradiated D-Ammonia (ND3 and its first use in a high energy photon beam, Nucl. Instrum. Methods Phys. Res. Sect. Accel. Spectrometers Detect. Assoc. Equip. 227, 35 (1984).

[4]	L. G. DeMarco, A. S. Brill, and D. G. Crabb, Stability of H, D, 14N, and 15N atoms in solid ammonia above 100 K, J. Chem. Phys. 108, 1423 (1998).

[5]	W. Meyer, Ammonia as a polarized solid target material—a review, Nucl. Instrum. Methods Phys. Res. Sect. Accel. Spectrometers Detect. Assoc. Equip. 526, 12 (2004).

[6]	P. M. McKee, Observations of radiation damage and recovery in ammonia targets, Nucl. Instrum. Methods Phys. Res. Sect. Accel. Spectrometers Detect. Assoc. Equip. 526, 60 (2004).

[7]	E. Long and K. Slifer, Dynamic Nuclear Polarization with Solid-State Millimeter Waves, 3D Printed Components, and Software-Defined-Radio-Based NMR, in Proceedings of The 18th International Workshop on Polarized Sources, Targets, and Polarimetry — PoS(PSTP2019), Vol. 379 (SISSA Medialab, 2020), p. 062.

[8]	C. Lama and E. Long, New Helium Recapture and Reliquefier System for Dynamic Nuclear Polarization at UNH., in Proceedings of 20th International Workshop on Polarized Source, Targets, and Polarimetry — PoS(PSTP2024), Vol. 472 (SISSA Medialab, 2025), p. 031.

[9]	D. Ruth and N. Santiesteban, RF Techniques for Enhancing Tensor Polarization in Solid Targets, in Proceedings of 20th International Workshop on Polarized Source, Targets, and Polarimetry — PoS(PSTP2024), Vol. 472 (SISSA Medialab, 2025), p. 032.

[10]	M. McClellan and E. Long, Complex deuteron NMR signals, Eur. Phys. J. A 61, 176 (2025).

## Appendix II: Tentative Research Team

| Experiment Participation | First Name | Last Name | Email | Primary Affiliation | Secondary Affiliation (if applicable) | Career Level | Tentative Role |
| --- | --- | --- | --- | --- | --- | --- | --- |
| On-site | Elena | Long | Elena.Long@unh.edu | University of New Hampshire (UNH) |  | 10 | 1, 3 |
| On-site | Karl | Slifer | Karl.Slifer@unh.edu | UNH |  | 11 | 1, 3 |
| On-site | Nathaly | Santiesteban | Nathaly.Santiesteban@unh.edu | UNH |  | 9 | 1, 3 |
| On-site | Chhetra | Lama | Chhetra.Lama@unh.edu | UNH |  |  | 1 |
| On-site | Aden | Whitney | Aden.Whitney@unh.edu | UNH |  |  | 1 |
| On-site | Muhammad | Farooq | Muhammad.Farooq@unh.edu | UNH |  |  | 1 |
| On-site | Anchit | Arora | Anchit.Arora@unh.edu | UNH |  |  | 1, 2 |

Definitions of experiment participation:

-	On-site: individuals who will travel to the BeamNetUS facility to perform or supervise hands on work in the laboratory before or during the scheduled experiment

-	Remote support: can vary by team but may include individuals who perform remote data analysis, supervision, coordination, or observation

-	Not attending: individuals involved in other aspects such as pre-experiment design or post-experiment analysis and interpretation

Please select from the following options for Career Level and Tentative Roll options:

Career Level:

1)	Undergraduate Student

2)	Graduate Student

3)	University Postdoctoral Researcher/Fellow/Scholar

4)	University Research Associate

5)	University Scientist

6)	Laboratory Postdoctoral Researcher/Fellow/Scholar

7)	Laboratory Research Associate

8)	Laboratory Scientist

9)	Assistant (Research) Professor

10)	Associate (Research) Professor

11)	(Research) Professor

12)	Other

Tentative Role:

1)	Experimental

2)	Theory and Simulations

3)	Supervision and Coordination

4)	Other (please specify)

Lead PI Elena Long is an Associate Professor at the University of New Hampshire with expertise in spin-polarized nuclear physics experiments. She is the lead Spokesperson on Jefferson Lab experiment E12-15-005 to measure the elastic and quasi-elastic tensor spin observables Azz and T20. Co-PI Karl Slifer  is the lead Spokesperson on Jefferson Lab experiment E12-013-011 to measure b1 tensor structure function. Co-PI S. Nathaly Santiesteban is the lead Spokesperson on Jefferson Lab LOI12-25-002 to measure exclusive electro-disintegration of tensor polarized deuterium. All three PIs have constructed and collaboratively operate a dynamic nuclear polarization (DNP) target lab at the University of New Hampshire, consisting of a 5T superconducting magnet, 1K helium evaporation refrigerator, 140 GHz microwave and electron paramagnetic resonance (EPR) system, NMR measurement system, secondary RF system for enhancing tensor polarization, and a 40 L/day helium recapture and reliquification system [7–9]. The UNH Polarized Target Lab was started by PI Slifer.  The PIs will primarily be responsible for optimizing the run conditions for each day of beam time following the previous day’s results, supporting beam running operations including material swapping and irradiation cryostat operations, as well as supervising and mentoring the graduate student researchers. PIs Long and Slifer and graduate student Anchit Arora have previous experience irradiating NH3, ND3, and deuterated butanol at the NIST MIRF accelerator, which has been used for recently published polarization results [10].

Graduate students Chhetra Lama, Aden Whitney, Muhammad Farooq, and Anchit Arora have all participated in multiple DNP experimental runs at UNH to measure maximum polarization on various target materials and optimize tensor polarization on ND3 and deuterated butanol. C. Lama and A. Whitney will primarily be in charge of the microwave frequency sweeps and DNP maximum polarization measurements during the evening shift. M. Farooq and A. Arora will primarily be in charge of continuing the DNP maximum polarization measurements on owl shift, as well as nightly analysis of results to pass on to the PIs in the morning for the following day’s beam operations.

The PIs also have closely collaborated with the Jefferson Lab Target Group, particularly with Chris Keith, James Maxwell, and James Brock, who will operate the irradiation cryostat and on-site DNP target with support from the PIs and the UNH graduate students. The PIs will also be working closely with Matt Poelker, who runs the UITF facility.

If travel support is available, we request $1400 for researcher travel to Jefferson Lab during operations, which includes flights and a truck rental for material transportation.

## Appendix III: Technical Parameter Table

| Parameter | Description | Comments/References |
| --- | --- | --- |
| Primary Measurements | Maximum polarization, T1 relaxation rates, Microwave frequency sweeps |  |
| Secondary Measurements | Temperature of and dose on material during irradiations |  |
| Beam Type | Electron |  |
| Beam Energy | 8 MeV |  |
| Charge | 6 C total |  |